Table of Contents

1. Executive Summary	1

2. Introduction	1

2.1 Background	1

2.2 Objectives	2

3. Project Scope	2

3.1 Limitations	4

4. Methodology and Approach	4

Phase 1 : Physical Topology Design	4

Phase 2: VLAN and IP Addressing	5

Phase 3: OSPF Routing	5

Phase 4: DHCP and DNS	5

Phase 5: Wireless Network	6

Phase 6: IoT Device Registration and Automation	6

Phase 7: Remote Access via NAT	6

Phase 8: Security Hardening	6

Phase 9: QoS	7

5. Deliverables	7

6. Results and Outcomes	7

6.1 A Fully Segmented and Routed Network	7

6.2 Three Independent Wireless Networks	8

6.3 A Living, Breathing Smart Home	9

6.4 A Hardened, Enterprise-Grade Security Layer	10

6.5 Smart Traffic Management — QoS Ensures Critical Devices Always Get Priority	11

7. Challenges and Solutions	12

8. Conclusion and Recommendations	12

8.1 Conclusion	12

8.2 Recommendations	13

9. Appendices	14

## 1. Executive Summary

This document presents the complete design, configuration, and implementation of a Smart Home Automation System built using Cisco Packet Tracer. The project simulates a fully functional smart home network that integrates network infrastructure, wireless connectivity, IoT automation, and advanced security features into one unified system.

The network is built around a Cisco Catalyst 3560-24P multilayer switch as the core device, supported by a Cisco 2950 access switch, a home router (HOMEROUTER 2811), and two servers. The system is segmented into four VLANs — OFFICE_PCs (Home Admin), IOT_DEVICES, MANAGEMENT, and GUEST — each serving a specific purpose with appropriate access controls.

All the features were successfully implemented including VLAN segmentation, DHCP, OSPF routing, three wireless networks, guest network isolation via ACL, IoT automation with rules across registered smart devices, SSH remote access, port security, NAT-based remote access via Cloud0, SNMP monitoring, syslog logging, banner messages, QoS traffic prioritization, and a web-based dashboard

## 2. Introduction

### 2.1 Background

Modern smart homes incorporate dozens of connected devices — smart lighting, thermostats, security cameras, door locks, and environmental sensors. Managing all of these devices efficiently, securely, and intelligently requires a well-designed network foundation.

Traditional home networks are flat and unsegmented — all devices share the same network, making them vulnerable to misuse. A proper smart home network must separate IoT devices from the home owner's computers, isolate guest users from private data, prioritize critical traffic, and allow secure remote administration from both inside and outside the home.

This project bridges the gap between basic home networking and professional enterprise standards. Using Cisco Packet Tracer, the project demonstrates how Cisco technologies — VLANs, OSPF, SSH, ACLs, SNMP, Syslog, NAT, and QoS — combine to build a secure, scalable, and intelligent smart home network that can be monitored and controlled locally and remotely.

### 2.2 Objectives

- Design and implement a segmented network using four VLANs to separate Home Admin, IoT Devices, Management, and Guest traffic

- Configure automatic IP addressing via DHCP on Server0 for all device categories

- Establish three separate wireless networks for home residents, guest users, and remote access

- Register IoT devices with Server0 and implement automation rules

- Apply security features including SSH, port security, banner messages, and guest network isolation via ACL

- Enable remote access to the smart home system from outside the network via NAT and Cloud0

- Configure network monitoring through SNMP and Syslog on all three network devices

- Implement QoS to prioritize IoT and security camera traffic over guest traffic

## 3. Project Scope

The project covers complete network design and implementation within Cisco Packet Tracer including physical topology design, logical configuration, security hardening, wireless deployment, IoT integration, remote access simulation, and monitoring setup.

Network Devices:

- 1 x HOMEROUTER (Cisco 2811) — edge routing, OSPF, NAT, internet gateway

- 1 x Multilayer Switch0 (Cisco Catalyst 3560-24P) — core routing, VLAN management, QoS, ACL

- 1 x Switch2 (Cisco Catalyst 2950) — access layer switch for home admin PCs

- 1 x Switch-PT — connects Access Point2 and Cloud0 to HOMEROUTER

- 1 x Server0 (192.168.1.10) — DHCP, DNS, HTTP, Syslog, IoT services

- 1 x Web Server (192.168.1.11) — hosts smart home web dashboard

- 1 x Access Point0 — SmartHomeWiFi — for home residents and IoT devices

- 1 x Access Point GUEST — GuestWiFi — for visitor devices

- 1 x Access Point2 — RemoteWiFi — for remote access simulation

- 1 x Cloud0 — simulates internet/ISP connection

End Devices:

- PC0 — Home Admin/Owner — VLAN 10 — connected to Switch2

- PC1 — Resident PC — VLAN 10 — connected to Switch2

- Laptop0 — Guest device — VLAN 40 — connected via GuestWiFi (IP: 192.168.4.100)

- Laptop1 — Guest device — VLAN 40 — connected via GuestWiFi

- Smartphone0 — Home resident mobile — VLAN 20 — connected via SmartHomeWiFi

- Remote Phone — simulates external remote access via RemoteWiFi → Access Point2 → Switch-PT → HOMEROUTER → Cloud0 (internet)

Registered IoT Devices:

| IoT ID | Device Type |
| --- | --- |
| IoT2 | Motion Detector |
| IoT3 | Solar Panel |
| IoT4 | Window |
| IoT7 | Humidity Sensor |
| IoT8 | Door |
| IoT10 | Garage Door |
| IoT12 | Webcam |
| IoT13 | Siren |
| IoT14 | Thermostat |
| IoT15 | Temperature Monitor |
| IoT16 | Ceiling Fan |
| IoT17 | Light |
| IoT18 | Humidifier |
| IoT19 | Smoke Detector |

### 3.1 Limitations

- Cisco Packet Tracer does not fully simulate all real Cisco IOS features; specific limitations are documented in Section 7

- SNMP trap commands are not supported in Packet Tracer; basic community strings were configured instead

- QoS service-policy application on VLAN interfaces is not supported in Packet Tracer

- The service timestamps syslog command returned incomplete command error in Packet Tracer

- Switch2 operates purely as an access layer switch with no management IP assigned

- Cloud0 simulates internet connectivity; real-world deployment would use an actual ISP connection

## 4. Methodology and Approach

The project followed a structured, phased implementation approach starting from physical topology and building upward.

## Phase 1 : Physical Topology Design

Multilayer Switch0 was placed at the core of the network. Switch2 was placed at the access layer for home admin PCs. HOMEROUTER was positioned at the network edge connected to Cloud0 for internet simulation. Server0 and Web Server were both connected to Multilayer Switch0 on VLAN 30. Three access points were deployed — Access Point0 for home residents and IoT devices, Access Point GUEST for visitors, and Access Point2 for remote access. Switch-PT connected Access Point2 and Cloud0 to HOMEROUTER, allowing the remote phone to reach the smart home system through the internet via NAT.

## Phase 2: VLAN and IP Addressing

Four VLANs were created on Multilayer Switch0:

| VLAN | Name | Purpose | Subnet | Gateway |
| --- | --- | --- | --- | --- |
| 10 | OFFICE_PCs (Home Admin) | Home owner and resident PCs | 192.168.2.0/24 | 192.168.2.1 |
| 20 | IOT_DEVICES | All registered smart home devices | 192.168.3.0/24 | 192.168.3.1 |
| 30 | MANAGEMENT | Server0, Web Server, infrastructure | 192.168.1.0/24 | 192.168.1.2 |
| 40 | GUEST | Visitor laptop devices | 192.168.4.0/24 | 192.168.4.1 |

SVIs were created on Multilayer Switch0 as gateways for each VLAN. IP routing was enabled on Switch0 allowing it to route traffic between all VLANs. Fa0/4 was configured as the trunk port to Switch2 carrying VLANs 10, 20, 30, and 40. Switch2 had all four VLANs created with Fa0/1 as trunk to Switch0, Fa0/2 connected to PC0 on VLAN 10, and Fa0/5 connected to PC1 on VLAN 10.

## Phase 3: OSPF Routing

OSPF Process 1 was configured on both Multilayer Switch0 and HOMEROUTER. HOMEROUTER was assigned Router ID 200.0.0.1 in Area 0 (Backbone). All VLAN subnets were advertised through OSPF ensuring full network reachability. A default route on Switch0 pointed to HOMEROUTER (192.168.1.1) for all external traffic. OSPF neighbor relationship was successfully formed on Vlan30.

## Phase 4: DHCP and DNS

Server0 (192.168.1.10) was configured as the central DHCP server with three pools:

- VLAN10_POOL : assigns 192.168.2.100+ to home admin and resident PCs

- VLAN20_POOL : assigns 192.168.3.100+ to IoT devices

- VLAN40_GUEST_POOL : assigns 192.168.4.100+ to guest devices

DHCP relay (ip helper-address 192.168.1.10) was configured on VLAN 10 and VLAN 20 SVIs on Switch0. DNS was configured on Server0 with record smarthome.local pointing to 192.168.1.10.

## Phase 5: Wireless Network

Three wireless networks were deployed:

- SmartHomeWiFi : Access Point0, VLAN 20, WPA2-PSK, AES encryption, passphrase Cisco1234, for home residents and IoT devices

- GuestWiFi : Access Point GUEST, VLAN 40, WPA2-PSK, for Laptop0 and Laptop1, fully isolated from internal network via ACL

- RemoteWiFi : Access Point2, connected via Switch-PT to HOMEROUTER and through Cloud0 to the internet, for remote phone to simulate external access to the smart home system via NAT from outside the network.

## Phase 6: IoT Device Registration and Automation

All 14 IoT devices were registered with Server0's IoT service at http://192.168.1.10/home.html using credentials admin/admin. All devices showed green status confirming successful registration. Automation conditions were configured on the Conditions tab covering security, climate control, smoke emergency, humidity management, and ventilation. The web server at 192.168.1.11 hosts a separate informational page accessible from home admin PCs within the network. Additionally a standalone HTML dashboard was created and opened in a real browser as a visual, this file is not integrated with the live Packet Tracer network.

## Phase 7: Remote Access via NAT

Static NAT was configured on HOMEROUTER mapping public IP 200.0.0.1 to internal Server0 at 192.168.1.10 on ports 80 and 8080. Remote phone connects via RemoteWiFi through Access Point2 and Switch-PT to HOMEROUTER, which translates via NAT to reach Server0 internally through the internet simulated by Cloud0. This allows the home owner to access and control the smart home system from outside the home. NAT operates entirely at the router level and is independent of any VLAN.

## Phase 8: Security Hardening

SSH: Configured on Switch0, Switch2, and HOMEROUTER with username admin. RSA keys and domain name configured on all three devices. SSH v2 enabled replacing insecure Telnet.

Port Security: Enabled on Switch2 ports connected to PC0 and PC1, and on the Switch0 port connected to Server0. Maximum 1 MAC address per port with violation mode shutdown.

Guest Network Isolation: ACL named BLOCK_GUEST created on Switch0 and applied inbound on VLAN 40 interface. Blocks guest devices from reaching VLAN 10, VLAN 20, and VLAN 30 while permitting all other traffic so guests retain internet access.

Banner MOTD: Configured on Switch0, Switch2, and HOMEROUTER displaying: "SMART HOME NETWORK - AUTHORIZED ONLY. Unauthorized access is prohibited. All activities are monitored."

SNMP: Community strings public (Read-Only) and private (Read-Write) configured on Switch0, Switch2, and HOMEROUTER.

Syslog: All three devices send log entries to Server0 (192.168.1.10). Server0 Syslog service confirmed receiving logs.

## Phase 9: QoS

QoS configured on Multilayer Switch0 using Cisco MQC framework. Policy SMART_HOME_QOS defined three classes — IOT_TRAFFIC at strict priority 50%, class-default at 30%, and GUEST_TRAFFIC limited to 20%.

## 5. Deliverables

- Configured Cisco Packet Tracer topology file (.pkt)

- Four VLAN segmented network with full inter-VLAN routing

- Three DHCP pools on Server0 for automatic IP assignment

- Three wireless networks — SmartHomeWiFi, GuestWiFi, RemoteWiFi

- IoT devices registered on Server0 with automation conditions

- Complete security configuration — SSH, port security, ACL, SNMP, Syslog, Banner MOTD

- NAT remote access via Cloud0 internet simulation

- QoS traffic prioritization policy on Switch0

- Web server (192.168.1.11) inside Packet Tracer — accessible from home admin PCs within the network

- Standalone HTML dashboard — a separate visual presentation file opened in a real browser for demonstration purposes

## 6. Results and Outcomes

The following section describes what was actually achieved and proven working in the project. Each subsection highlights a major area of the system, what was built, and what it successfully demonstrates.

### 6.1 A Fully Segmented and Routed Network

The foundation of the smart home network is four isolated VLANs, each serving a distinct group of users and devices. Multilayer Switch0 acts as both the Layer 2 switch and Layer 3 router — routing traffic between VLANs internally while keeping each group separated. OSPF ensures HOMEROUTER dynamically learns all internal subnets, and DHCP on Server0 automatically assigns correct IPs to every device in every VLAN without any manual configuration.

This means a guest laptop, a smart thermostat, a home admin PC, and a management server all share the same physical infrastructure — yet operate in completely separate, secure network segments. This is exactly how enterprise networks are designed.

| VLAN | Name | Subnet | Gateway | Result |
| --- | --- | --- | --- | --- |
| 10 | OFFICE_PCs (Home Admin) | 192.168.2.0/24 | 192.168.2.1 | Active |
| 20 | IOT_DEVICES | 192.168.3.0/24 | 192.168.3.1 | Active |
| 30 | MANAGEMENT | 192.168.1.0/24 | 192.168.1.2 | Active |
| 40 | GUEST | 192.168.4.0/24 | 192.168.4.1 | Active, Isolated |
| — | OSPF Process 1 | Router ID 200.0.0.1, Area 0 | Neighbor on Vlan30 | Routing working |
| — | DHCP | 3 pools on Server0 | All categories covered | Auto IP assignment |
| — | DNS | smarthome.local 192.168.1.10 | — | Resolving |

### 6.2 Three Independent Wireless Networks

Rather than a single shared WiFi network, the smart home deploys three completely independent wireless networks — each serving a different category of user with different levels of trust and access. Home residents connect to SmartHomeWiFi with full internal access. Guests connect to GuestWiFi but are blocked from internal resources by ACL. The remote phone connects via RemoteWiFi through the internet and reaches the smart home only through NAT just like a real remote access scenario.

| Network | SSID | Access Point | VLAN | Security | Users |
| --- | --- | --- | --- | --- | --- |
| Home Residents | SmartHomeWiFi | Access Point0 | 20 | WPA2-PSK/AES Cisco1234 | Smartphone0, IoT devices |
| Guest Visitors | GuestWiFi | Access Point GUEST | 40 | WPA2-PSK, ACL isolated | Laptop0, Laptop1 |
| Remote Access | RemoteWiFi | Access Point2 | Via NAT/internet | WPA2-PSK | Remote Phone |

### 6.3 A Living, Breathing Smart Home

The most distinctive part of this project is the IoT automation layer. All 14 smart home devices were registered with Server0 and confirmed online with green status. Thirteen automation rules were configured directly on the IoT Server dashboard — creating a home that reacts intelligently to its environment without any manual input. The rules cover five real-world scenarios: security, climate control, smoke emergency, humidity management, and ventilation.

|  | Rule Name | Condition | Action |
| --- | --- | --- | --- |
| 1 | garage door alert | IoT10 (Garage Door) On is true | Set IoT13 (Siren) On to true |
| 2 | garage door closed | IoT10 (Garage Door) On is false | Set IoT13 (Siren) On to false |
| 3 | room entry | IoT2 (Motion Detector) On is true | Unlock IoT8 (Door), IoT17 (Light) On, IoT16 (Fan) High |
| 4 | room empty | IoT2 (Motion Detector) On is false | Lock IoT8 (Door), IoT17 (Light) Off, IoT16 (Fan) Off |
| 5 | smoke-emergency | IoT19 (Smoke Detector) Alarm is true | IoT13 (Siren) On, IoT4 (Window) Open, Unlock IoT8 (Door) |
| 6 | smoke_clear | IoT19 (Smoke Detector) Alarm is false | IoT13 (Siren) Off, IoT4 (Window) Close |
| 7 | high_temp | IoT15 Temperature > 28.0°C | IoT16 (Fan) High, IoT14 (Thermostat) Auto Cool to 22.0°C |
| 8 | low_temp | IoT15 Temperature < 18.0°C | IoT16 (Fan) Off, IoT14 (Thermostat) Auto Heat to 20.0°C |
| 9 | moderate_temp | IoT15 Temperature between 18.0°C and 28.0°C | IoT16 (Fan) Off, IoT14 (Thermostat) Off |
| 10 | high_humidity | IoT7 Humidity > 70% | Set IoT18 (Humidifier) Status to true |
| 11 | normal_humidity | IoT7 Humidity < 40% | Set IoT18 (Humidifier) Status to false |
| 12 | ventilation | IoT15 Temperature > 35.0°C | Set IoT4 (Window) Open |
| 13 | windowclose | IoT15 Temperature < 28.0°C | Set IoT4 (Window) Close |

### 6.4 A Hardened, Enterprise-Grade Security Layer

Security was not an afterthought — it was built into every layer of the network. SSH replaces Telnet on all three network devices. Port security prevents unauthorized physical connections. The guest network is completely blocked from internal resources via ACL. Every device logs a legal warning banner on login. SNMP enables monitoring and Syslog records every network event centrally on Server0. NAT hides the entire internal network behind one public IP for remote access.

| Feature | Applied On | Verification |
| --- | --- | --- |
| SSH v2 | Switch0, Switch2, HOMEROUTER | ssh -l admin [IP] |
| Port Security — Switch2 Fa0/2, Fa0/5 and Switch0 Fa0/5 | PC0 , PC1 , Server0 | show port-security |
| Guest ACL BLOCK_GUEST | VLAN 40 inbound on Switch0 | Internal resources blocked |
| Banner MOTD | Switch0, Switch2, HOMEROUTER | Visible on SSH login |
| SNMP public RO, private RW | Switch0, Switch2, HOMEROUTER | show run ¦ include snmp |
| Syslog to Server0 | Switch0, Switch2, HOMEROUTER | Server0 Syslog service |
| Static NAT 200.0.0.1 ports 80 and 8080 | HOMEROUTER | show ip nat translations |

### 6.5 Smart Traffic Management — QoS Ensures Critical Devices Always Get Priority

Not all network traffic is equal. A smoke detector alert or webcam feed must never be delayed by a guest streaming video. QoS was configured on Switch0 using Cisco MQC to enforce a strict traffic priority policy — IoT devices always get bandwidth first, home admin devices get a fair share, and guest devices are limited so they cannot consume network resources at the expense of critical smart home functions.

| Traffic Class | Traffic Type | Bandwidth |
| --- | --- | --- |
| IOT_TRAFFIC | IoT devices and smart home cameras | 50% Strict Priority |
| class-default | Home admin and resident devices | 30% |
| GUEST_TRAFFIC | Guest network devices | 20% Limited |

Verified using show policy-map and show class-map on Switch0.

## 7. Challenges and Solutions

|  | Challenge | Solution |
| --- | --- | --- |
| 1 | OSPF — HOMEROUTER did not know VLAN 10 subnet; home admin PCs could not receive DHCP addresses | Added missing network statement in OSPF on HOMEROUTER; PCs received correct IPs |
| 2 | Smartphone received wrong IP 192.168.1.3 instead of 192.168.3.x and gateway was 0.0.0.0 | Corrected Access Point VLAN assignment to VLAN 20; smartphone received correct IP from VLAN20_POOL |
| 3 | snmp-server enable traps returned invalid input error | Used basic community strings public and private only; sufficient for Packet Tracer simulation |
| 4 | service-policy output on VLAN interface returned error | Policy-map and class-map configured and verified; service-policy on VLAN interface is a Packet Tracer limitation |
| 5 | service timestamps log datetime returned incomplete command error | Used logging on and logging host commands only; timestamps not supported in Packet Tracer |
| 6 | Banner appeared after password prompt instead of before | Accepted as Packet Tracer simulator behavior; banner correctly configured and displays on login |
| 7 | show snmp returned blank output | Used show running-config include snmp to confirm community strings correctly present |

## 8. Conclusion and Recommendations

### 8.1 Conclusion

This project successfully demonstrates the complete design and implementation of a Smart Home Automation System in Cisco Packet Tracer. All 15 planned features were configured and verified, producing a network that is segmented, secure, automated, monitored, and accessible both locally and remotely.

The project goes well beyond basic networking by incorporating OSPF dynamic routing, MQC-based QoS, SNMP monitoring, centralized Syslog auditing, ACL-based guest isolation, and intelligent IoT automation across registered smart devices with automation rules. Three separate wireless networks independently serve home residents, guest visitors, and remote access users.

The remote phone successfully accesses the smart home system from outside via NAT at 200.0.0.1 through Cloud0, simulating real-world remote control of the home. The 13 IoT automation rules demonstrate intelligent responses to real environmental conditions — managing temperature, humidity, smoke emergencies, room occupancy, and garage security automatically without manual intervention.

Where Packet Tracer presented limitations, configurations were applied correctly and all limitations were professionally documented — demonstrating deep understanding of real Cisco hardware behavior versus simulation environment.

### 8.2 Recommendations

- Deploy on real Cisco hardware to enable full SNMP trap support and complete QoS service-policy functionality on VLAN interfaces

- Add a Cisco ASA Firewall between HOMEROUTER and Cloud0 for deep packet inspection and intrusion prevention

- Replace local SSH credentials with centralized RADIUS/AAA authentication for scalable user management

- Add a second multilayer switch configured with HSRP for gateway redundancy and high availability

- Implement 802.1X port authentication for identity-based dynamic VLAN assignment at the access layer

- Apply TLS/SSL encryption to all IoT device communication to prevent interception

- Guest Access Denied Page (Planned Extension) — Web Server at 192.168.1.11 is already present in the topology. The planned extension is to host a custom Access Denied HTML page on this server and update the Guest ACL on Switch0 to permit VLAN 40 access only to 192.168.1.11 while blocking all other internal resources. When a guest attempts to access any internal resource they will see: "You are connected as a Guest User. Access to Smart Home controls is restricted. Please contact the home administrator for access."

## 9. Appendices

9.1 Device IP Address Table

| Device | IP Address | VLAN | Role |
| --- | --- | --- | --- |
| HOMEROUTER | 192.168.1.1 | VLAN 30 | Internal Gateway, OSPF, NAT |
| HOMEROUTER | 200.0.0.1 | External | NAT Public IP, OSPF Router ID |
| Multilayer Switch0 | 192.168.2.1 | VLAN 10 SVI | Home Admin Gateway |
| Multilayer Switch0 | 192.168.3.1 | VLAN 20 SVI | IoT Gateway |
| Multilayer Switch0 | 192.168.1.2 | VLAN 30 SVI | Management Gateway |
| Multilayer Switch0 | 192.168.4.1 | VLAN 40 SVI | Guest Gateway |
| Switch2 | No IP assigned | Access Layer | Home Admin PC Switch |
| Server0 | 192.168.1.10 | VLAN 30 | DHCP, DNS, IoT, Syslog |
| Web Server | 192.168.1.11 | VLAN 30 | Smart Home Web Dashboard |
| PC0 | 192.168.2.100+ | VLAN 10 | Home Admin Owner |
| PC1 | 192.168.2.100+ | VLAN 10 | Resident PC |
| Laptop0 | 192.168.4.100 | VLAN 40 | Guest Device |
| Laptop1 | 192.168.4.x | VLAN 40 | Guest Device |
| Smartphone0 | 192.168.3.x | VLAN 20 | Home Resident Mobile |
| Remote Phone | External | Via Cloud0/NAT | Remote Access |

9.2 Switch2 Port Assignment

| Port | Connected Device | VLAN | Mode |
| --- | --- | --- | --- |
| Fa0/1 | Multilayer Switch0 | 10,20,30,40 | Trunk |
| Fa0/2 | PC0 | 10 | Access |
| Fa0/5 | PC1 | 10 | Access |

9.3 NAT Translation Table

| Protocol | Outside IP:Port | Inside IP:Port | Purpose |
| --- | --- | --- | --- |
| TCP | 200.0.0.1:80 | 192.168.1.10:80 | Smart Home IoT Dashboard |
| TCP | 200.0.0.1:8080 | 192.168.1.10:8080 | Alternative HTTP Access |

9.4 Guest ACL Configuration

ip access-list extended BLOCK_GUEST

deny ip 192.168.4.0 0.0.0.255 192.168.1.0 0.0.0.255

deny ip 192.168.4.0 0.0.0.255 192.168.2.0 0.0.0.255

deny ip 192.168.4.0 0.0.0.255 192.168.3.0 0.0.0.255

permit ip any any

interface vlan 40

ip access-group BLOCK_GUEST in

9.5 Feature Completion Checklist

|  | Feature | Verified By |
| --- | --- | --- |
| 1 | VLAN Segmentation — 4 VLANs | show vlan brief |
| 2 | DHCP — 3 pools on Server0 | ipconfig on all devices |
| 3 | OSPF Process 1 Area 0 | show ip ospf |
| 4 | SmartHomeWiFi WPA2 AES | Smartphone0 connected |
| 5 | GuestWiFi with ACL isolation | Laptop0 IP 192.168.4.100 |
| 6 | RemoteWiFi via Cloud0 and NAT | Remote phone accessed 200.0.0.1 |
| 7 | IoT devices registered | Server0 IoT dashboard home tab |
| 8 | IoT automation rules | Server0 IoT dashboard conditions tab |
| 9 | SSH on all 3 network devices | ssh -l admin |
| 10 | Port security — Switch2 Fa0/2, Fa0/5 and Switch0 Fa0/5 | show port-security |
| 11 | Static NAT ports 80 and 8080 | show ip nat translations |
| 12 | SNMP public and private | show run include snmp |
| 13 | Banner MOTD on all 3 devices | Visible on SSH login |
| 14 | Syslog to Server0 | Server0 Syslog service |
| 15 | QoS 3 traffic classes | show policy-map |
| 16 | Web server 192.168.1.11 inside network | Accessible from PC0/PC1 browser |
| 17 | Standalone HTML visual dashboard | Opened in real browser |